A S S I G N A B L E    S E Q U E N T I A L

I / O    D E V I C E

ABSTRACT:

ASDEV IS A PSEUDO DEVICE FOR RSX11D WHICH LOOKS TO

A USER LIKE A TERMINAL. IT CAN DO SEQUENTIAL I/O IN EITHER DIRECTION

BUT DOES NOT APPEAR TO BE A DIRECTORY DEVICE.

IN REALITY, HOWEVER, ASDEV ALLOWS INPUT OR OUTPUT (OR BOTH)

TO BE ASSOCIATED WITH (NORMALLY ASCII) FILES ON ANY RSX DATASET,

SO THAT I/O MAY BE FORCED INTO THE FILE SYSTEM EVEN WHERE IT WAS

NOT ORIGINALLY SO PROGRAMMED.

THE MCR TASK "ASN" CONTROLS THE ASSIGNMENTS BY PASSING A

CHARACTER STRING OF THE FILE SPECS TO ASDEV VIA A SPECIAL QIO$.

THE DEVICE HANDLER TAKES IT FROM THERE.

THIS IS OBVIOUSLY USEFUL WHERE IT IS DESIRED TO FORCE

A TASK'S PRINTER OUTPUT TO SPOOL WHERE THE TASK NORMALLY WOULD

USE QIO AND MESS UP THE REST OF THE USERS. THE TASK MAY BE USED

AS A VIRTUAL TERMINAL TOO. "RUN ...MCR/TI=AS0" WOULD PLAY THE

ROLE OF CONTROL-C, AND INPUT AND OUTPUT FILES WOULD ACT AS THE

CONSOLE, WHICH COULD THEN INPUT DATA (FOR USE WITH FORTRAN

PROGRAMS THAT DON'T USE GET COMMAND LINE).

WRITE-UP:

ASDEV WILL PASS VARIABLE LENGTH DATA TO A USER PROGRAM OR FROM

ONE WITH RECORD LENGTHS UP TO 84 CHARACTERS (IT TRUNCATES LONG RECORDS)

AND WILL ALLOW FILE CLOSE/OPEN FROM THE CONSOLE (OR AT HANDLER UNLOAD).

IF NO FILE IS ASSIGNED ON EITHER SIDE, ASDEV ACTS AS A GARBAGE

DEVICE, I.E., IT DISCARDS OUTPUT AND GIVES IMMEDIATE ENDFILE ON INPUT.

WHENEVER ASDEV SEES AN END FILE ON AN INPUT FILE, IT CLOSES

THE INPUT FILE, RETURNS EOF, AND SUBSEQUENTLY WILL GIVE INSTANT EOF.

OUTPUT ERROR IS NOT CHECKED SIMILARLY. IT IS ASSUMED YOUR

DISK WON'T RUN OUT OF SPACE. NO ERROR IS RETURNED AND ASDEV WILL

ACT AS IF THE QIO WENT THROUGH OK (IT ENTERS "GARBAGE MODE"). THIS

IS NOT NORMALLY A PROBLEM.

IF A NEW INPUT OR OUTPUT FILE IS ASSIGNED, THE OLD ONE IS

CLOSED FIRST. SPECIAL CLOSE COMMANDS ARE NOT REQUIRED.

IF SWITCH 14 IS UP (ON REAL OR RSX EMULATED SWITCH REGISTER

SWITCHES), ASDEV WILL DO A QIO TO LUN 2 (USUALLY A TERMINAL) AS WELL

AS ITS POSSIBLE FILE STRUCTURED OUTPUT. THIS MAKES IT POSSIBLE TO

USE ASDEV TO CAPTURE OUTPUT TO CO: WITHOUT SACRIFICING THE TIMELY

MESSAGES THE SYSTEM EMITS THERE.

THE SWITCH FEATURE MAY BE EXCLUDED VIA CONDITIONAL ASSEMBLY.

TO ASSEMBLE ASDEV, USE A STRING LIKE

MAC ASDEV=ASDEV

TKB

TKB>ASDEV/PR/-FP/-FX/-CP/-AB=ASDEV,[1,1]EXEC.STB/SS

/

TASK=AS....

UIC=[1,1]

STACK=300

PRI=200

POOL=60

LIBR=SYSRES:RO

UNITS=4

ASG=TT0:1:2

ASG=SY:3:4

PAR=GEN

//

ASDEV IS CONTROLLED BY THE MCR TASK "ASSIGN".

TO ASSEMBLE ASSIGN, USE A COMMAND STRING LIKE

MAC ASSIGN=ASSIGN

TKB>ASSIGN/MU/-FP/PR=ASSIGN

/

UIC=[1,1]

TASK=...ASN

PRI=70

UNITS=2

ASG=TI:1

ASG=AS:2

STACK=64

//

MAKE A PUD FOR AS0: THAT LOOKS LIKE A TERMINAL PUD

EXCEPT FOR DEVICE NAME AND UNIT AS0:. THEN INSTALL ASDEV AND

ASSIGN AND YOU'RE READY TO GO. LOAD AS: AND IT WILL RUN.

(TO MAKE A PUD, FIND A SPARE DEVICE OR DO A SYSGEN AND USE

THE SYS/DEV COMMAND, THEN THE OPEN ADDR/KNL COMMAND TO ALTER

THE PUD ENTRY FOR THE SPARE DEVICE.)

TO ASSIGN A DATASET, THE COMMAND FORMAT IS

MCR>ASN DV:[P,PN]FILENAME.TYP=IN		;FOR INPUT

OR

MCR>ASN DV:[P,PN]FILENAME.TYP=OUT		;FOR OUTPUT

ASN WILL NOT TELL YOU OF ANY ERRORS. IF YOU MISTYPE, REDO

THE COMMAND.

THE LISTING OF ASDEV GIVES THE QIO'S FOR DEVICE ASSIGNMENTS.

BASICALLY I/O CODE 	5 IS OPEN DATASET FOR INPUT

6 IS CLOSE INPUT DATASET

7 IS OPEN DATASET FOR OUTPUT, AND

8 IS CLOSE OUTPUT DATASET.

THE QIO USES THOSE CODES (2400,3000,3400, AND 4000) FOR

THE I/O CODE IN THE QIO, AND THE PARAMETER BLOCK CONTAINS THEE

ADDRESS AND BYTE COUNT OF A BUFFER CONTAINING THE ASCII FILE

SPECS. THESE SPECS ARE PARSED WITHIN THE HANDLER AND ALL IT

REQUIRES IS THE ASCII STRING ITSELF, WITH ITS BYTE COUNT.

THESE CODES ARE USED FOR DEVICE CONTROL IN OTHER DEVICES

AND WILL NORMALLY NEVER BE SEEN FOR ANYTHING ELSE.

GLENN C. EVERHART

NOV. 2, 1978