Command? R CPMUG-PROTOCOL

<CPMUG-PROTOCOL>   27-AUG-82

Parent: PROTOCOLS

MODEM PROTOCOL OVERVIEW  178 lines, 7.5K

1/1/82 by Ward Christensen.  I will maintain a master copy of

this.  Please pass on changes or suggestions via CBBS/Chicago

at (312) 545-8086, or by voice at (312) 849-6279.

NOTE this does not include things which I am not familiar with,

such as the CRC option implemented by John Mahr.

Last Rev: (none)

At the request of Rick Mallinak on behalf of the guys at

Standard Oil with IBM P.C.s, as well as several previous

requests, I finally decided to put my modem protocol into

writing.  It had been previously formally published only in the

AMRAD newsletter.

Table of Contents

1. DEFINITIONS

2. TRANSMISSION MEDIUM LEVEL PROTOCOL

3. MESSAGE BLOCK LEVEL PROTOCOL

4. FILE LEVEL PROTOCOL

5. DATA FLOW EXAMPLE INCLUDING ERROR RECOVERY

6. PROGRAMMING TIPS.

-------- 1. DEFINITIONS.

<soh>   01H

<eot>   04H

<ack>   05H

<nak>   15H

<can>   18H

-------- 2. TRANSMISSION MEDIUM LEVEL PROTOCOL

Asynchronous, 8 data bits, no parity, one stop bit.

The protocol imposes no restrictions on the contents of the

data being transmitted.  No control characters are looked for

in the 128-byte data messages.  Absolutely any kind of data may

be sent - binary, ASCII, etc.  The protocol has not formally

been adopted to a 7-bit environment for the transmission of

ASCII-only (or unpacked-hex) data , although it could be simply

by having both ends agree to AND the protocol-dependent data

with 7F hex before validating it.  I specifically am referring

to the checksum, and the block numbers and their ones-

complement.

.cp 16

Those wishing to maintain compatibility of the CP/M file

structure, i.e. to allow modemming ASCII files to or from CP/M

systems should follow this data format:

* ASCII tabs used (09H); tabs set every 8.

* Lines terminated by CR/LF (0DH 0AH)

* End-of-file indicated by ^Z, 1AH.  (one or more)

* Data is variable length, i.e. should be considered a

continuous stream of data bytes, broken into 128-byte

chunks purely for the purpose of transmission.

* A CP/M "peculiarity": If the data ends exactly on a

128-byte boundary, i.e. CR in 127, and LF in 128, a

subsequent sector containing the ^Z EOF character(s)

is optional, but is preferred.  Some utilities or

user programs still do not handle EOF without ^Zs.

* The last block sent is no different from others, i.e.

there is no "short block".

-------- 3. MESSAGE BLOCK LEVEL PROTOCOL

Each block of the transfer looks like:

<SOH><blk #><255-blk #><--128 data bytes--><cksum>

in which:

<SOH>       = 01 hex

<blk #>     = binary number, starts at 01 increments by 1, and

wraps 0FFH to 00H (not to 01)

<255-blk #> = blk # after going thru 8080 "CMA" instr, i.e.

each bit complemented in the 8-bit block number.

Formally, this is the "ones complement".

<cksum>     = the sum of the data bytes only.  Toss any carry.

-------- 4. FILE LEVEL PROTOCOL

---- 4A. COMMON TO BOTH SENDER AND RECEIVER:

All errors are retried 10 times.  For versions running with

an operator (i.e. NOT with XMODEM), a message is typed after 10

errors asking the operator whether to "retry or quit".

Some versions of the protocol use <can>, ASCII ^X, to

cancel transmission.  This was never adopted as a standard, as

having a single "abort" character makes the transmission

susceptible to false termination due to an <ack> <nak> or <soh>

being corrupted into a <can> and canceling transmission.

The protocol may be considered "receiver driven", that is,

the sender need not automatically re-transmit, although it does

in the current implementations.

.cp 13

---- 4B. RECEIVE PROGRAM CONSIDERATIONS:

The receiver has a 10-second timeout.  It sends a <nak>

every time it times out.  The receiver's first timeout, which

sends a <nak>, signals the transmitter to start.  Optionally,

the receiver could send a <nak> immediately, in case the sender

was ready.  This would save the initial 10 second timeout.

However, the receiver MUST continue to timeout every 10 seconds

in case the sender wasn't ready.

Once into a receiving a block, the receiver goes into a

one-second timeout for each character and the checksum.  If the

receiver wishes to <nak> a block for any reason (invalid

header, timeout receiving data), it must wait for the line to

clear.  See "programming tips" for ideas

Synchronizing:  If a valid block number is received, it

will be: 1) the expected one, in which case everything is fine;

or 2) a repeat of the previously received block.  This should

be considered OK, and only indicates that the receivers <ack>

got glitched, and the sender re-transmitted; 3) any other block

number indicates a fatal loss of synchronization, such as the

rare case of the sender getting a line-glitch that looked like

an <ack>.  Abort the transmission, sending a <can>

---- 4C. SENDING PROGRAM CONSIDERATIONS.

While waiting for transmission to begin, the sender has

only a single very long timeout, say one minute.  In the

current protocol, the sender has a 10 second timeout before

retrying.  I suggest NOT doing this, and letting the protocol

be completely receiver-driven.  This will be compatible with

existing programs.

When the sender has no more data, it sends an <eot>, and

awaits an <ack>, resending the <eot> if it doesn't get one.

Again, the protocol could be receiver-driven, with the sender

only having the high-level 1-minute timeout to abort.

.cp24

-------- 5. DATA FLOW EXAMPLE INCLUDING ERROR RECOVERY

Here is a sample of the data flow, sending a 3-block message.

It includes the two most common line hits - a garbaged block,

and an <ack> reply getting garbaged.  <xx> represents the

checksum byte.

SENDER                                  RECEIVER

times out after 10 seconds,

<---            <nak>

<soh> 01 FE -data- <xx> --->

<---            <ack>

<soh> 02 FD -data- xx   --->    (data gets line hit)

<---            <nak>

<soh> 02 FD -data- xx   --->

<---            <ack>

<soh> 03 FC -data- xx   --->

(ack gets garbaged)  <---            <ack>

<soh> 03 FC -data- xx   --->            <ack>

<eot>                   --->

<---            <ack>

-------- 6. PROGRAMMING TIPS.

* The character-receive subroutine should be called with a

parameter specifying the number of seconds to wait.  The

receiver should first call it with a time of 10, then <nak> and

try again, 10 times.

After receiving the <soh>, the receiver should call the

character receive subroutine with a 1-second timeout, for the

remainder of the message and the <cksum>.  Since they are sent

as a continuous stream, timing out of this implies a serious

like glitch that caused, say, 127 characters to be seen instead

of 128.

* When the receiver wishes to <nak>, it should call a "PURGE"

subroutine, to wait for the line to clear.  Recall the sender

tosses any characters in its UART buffer immediately upon

completing sending a block, to ensure no glitches were mis-

interpreted.

The most common technique is for "PURGE" to call the

character receive subroutine, specifying a 1-second timeout,

and looping back to PURGE until a timeout occurs.  The <nak> is

then sent, ensuring the other end will see it.

* You may wish to add code recommended by Jonh Mahr to your

character receive routine - to set an error flag if the UART

shows framing error, or overrun.  This will help catch a few

more glitches - the most common of which is a hit in the high

bits of the byte in two consecutive bytes.  The <cksum> comes

out OK since counting in 1-byte produces the same result of

adding 80H + 80H as with adding 00H + 00H.

-- No child messages --

Command? CPM

Type BINEX after the A0> prompt to return to BIN-EX

Type CHAT to chat with the system operator; BYE to quit

BIN-EX message texts are on USER 1, A: and B: drives

Files for downloading are on USER 0, both drives; DIR for directory

A0>USER 0

A0>DIR A:

ALIENS11.DOC   4k | ALIENS11.OBJ  14k | BACKUP  .ASM  28k | BACKUP  .DOC   8k

BINEX   .COM   2k | BY2-8250.ASM   8k | BY2-8251.ASM   8k | BY2-MMII.ASM   8k

BY2-SIO .ASM   8k | BYE     .COM   2k | BYEII   .ASM  52k | CAL     .OBJ   6k

CATALOG .      4k | CHAT    .COM   2k | CHESS   .OBJ   8k | DISK    .OBJ   4k

DISK74A .DOC   6k | DISK76  .ASM  48k | DOWNLOAD.HLP   6k | DU      .OBJ   6k

DU-V77  .DOC   6k | FIND    .COM   2k | FIND    .DOC   2k | FIND    .OBJ   2k

FIXTEX  .ASM  22k | HELP    .OBJ   4k | MBOOT   .ASM   8k | MBOOTVT .ASM   6k

MDM705  .DOC  34k | MDM705  .OBJ  18k | MDM705  .SET   8k | MDM705CF.ASM  14k

MDM705NM.ASM   4k | MDM705VT.DOC   4k | MDM705VT.OBJ  16k | MDMEQU02.DOC  12k

PACMAN  .DOC   2k | PACMAN  .OBJ  18k | PIPMODEM.ASM   4k | PIPMODEM.DOC   8k

PROT    .OBJ   4k | RCPM-038.LST  22k | SD      .OBJ   2k | SSUB    .OBJ   4k

USQ     .OBJ  10k | XMODEM  .COM   4k | XMODEM  .HLP   2k | Z2CON   .WQ   78k

Z80CHESS.DOC   6k | Z80CHESS.OBJ   8k | ZZERR   .DAT   2k | ZZMUMPS .DOC   2k

ZZMUMPS .OBJ  42k | ZZSETGLB.OBJ   2k | ZZSETMPS.OBJ   2k

Drive A, user 0 contains 616K in 55 files with 144K free

A0>TYP XMODEM.HLP

TYP?

A0>TYPE A:XMODEM.HLP

Ctrl-S pauses, Ctrl-C Aborts, Ctrl-X skips to next file

Listing file A:XMODEM.HLP

XMODEM is the downloading program on this system. It transfers binary

files block by block with error checking using either a CHECKSUM

or CRC. The protocol is known as the CPMUG or Christensen protocol.

It is compatible with several communications programs.

The syntax for its use is:

XMODEM S filename.typ    (to send a file from here to you)

or

XMODEM R filename.typ    (to receive a file from you)

Programs which are compatible include:

MDM705    PUBLIC DOMAIN     (available from this system)  CP/M

MODEM7      "      "

MODEM2      "      "

MODEM etc   "      "

MODEMIBM  PUBLIC DOMAIN     (available from this system) PCDOS

AMODEM    PUBLIC DOMAIN     (available from this system) ATARI

UMODEM    PUBLIC DOMAIN     (available from this suytem) UNIX

AMCALL    MicroCALL Services 9655-M Homestead Court, Laurel MD 20707

ASCOM     DMA or Westico  25 Van Zant Street, Norwalk CT 06855

CP/M-80, CP/M-86, PCDOS, MSDOS

M.I.T.E.  Mycroft Labs  PO Box 6045 Tallahassee, FL 32301

CP/M-80

COMMX     Hawkeye Grafix  23914 Mobile, Canoga Park, CA 91307

CP/M-80  $150 or $900 source (also available in

FORTRAN for use on mainframe) (213) 348-7909

PC-TALK III  The Headlands Press, PO BOX 862, Tiburon, CA 94920

for IBM-PC  suggested donation $35.00 (or freeware)

MODEM80   The Alternate Source, 705 N. Pennsylvania, Lansing, MI 48906

$39.85 TRS-80 MOD 1 & III Reviewed in May 1983 Creative Computing

ZTERM Professional    Southwestern Data Systems, PO Box 582, Santee CA 92071

$149   Apple softcard CP/M Review May 1983 Creative Computing

ASCII Express Professional Southwestern Data Systems - APPLE II 48K

LMODEM    (DEC-10)

UMODEM    (UNIX)     Written in C (available on this system)

VMODEM    (VAX-VMS)  a VAX version of UMODEM (available on this system)

XCHANGE-11 (PDP-11 RSTS-E)

A0>DIR B:

AMODEM42.ATR  10k | APMBOOT .ASM   8k | CAT     .OBJ   2k | CIS-DOW .MAC  14k

CIS-EXEC.AQM  20k | CIS-EXEC.DOC   4k | CIS-MNET.PRO  14k | CISINFO .DOC   2k

CISPROTO.DOC  18k | CRCK8   .OBJ   6k | DBASE   .NTS   6k | ELIZA   .BAS   8k

HELP    .HQP   2k | HELP2   .HQP  14k | HELP20  .AQM  24k | INDEX   .OBJ  10k

INTNET  .TXT   4k | LDIR    .OBJ   6k | LRUN    .OBJ   2k | LU      .DOC  24k

LU      .OBJ  18k | LUDEF1  .DOC   8k | MASTHEAD.ASM   6k | MDM705  .INF   6k

MDM705  .MSG   2k | MDM705AP.ASM  10k | MDM705DP.ASM   8k | MDM705HZ.ASM   8k

MDM705OS.ASM  14k | MDM705XE.ASM  10k | MENU    .DQC   4k | MENU    .OQJ  10k

MFT     .ASM  18k | MMDF    .PQO  22k | MNETCMD .TRS   2k | MODEM80 .TRS  12k

MODEMIBM.ASM  56k | NAPLPS  .DOC  22k | NCAT    .OBJ   8k | PC-APRG .DOC  12k

PCSERPRO.DQC  16k | PRLMOV  .ASM   4k | RAMDSK  .ASM  24k | RAMDSK  .DOC   4k

RAMDSK  .OBJ   2k | RESOURCE.OBJ   8k | REZ     .DOC  30k | SCRAMBLE.ASM   6k

SCRAMBLE.DOC   2k | SORTV   .DOC   2k | SORTV-12.MAC  14k | SYNONYM .ASM   6k

SYNONYM .DOC   4k | UMODEM  .DOC   4k | UMODEM28.C    38k | UMODEM28.HLP   8k

UNSPOOL .DOC  10k | UNSPOOL .MAC  18k | UNSPOOL .OBJ   2k | VMODEM28.C     8k

VMODEM28.H     2k | XREF    .BAS   6k

Drive B, user 0 contains 672K in 62 files with 28K free