This is SWITCH: subdirectory .MICOM

It contains MICOM control programs from Peabody of Chemistry Dept.

As modified for UW Physics Dept Micom Instanet6000 PABX Series 40

intelligent terminal switch by Jim Belonis

NOTE: there are TWO switches involved here, a homebrew 128 terminal

to 32 computer ports statistical multiplexer, and a MICOM 6000

SWITCH.HLP  Help on both switches.  See HELP SWITCH MICOM MICOM_PROGRAM

for Administrative port commands

MICOM program needs username SYSTEM to pass all commands.

Helps if SET TERMINAL/PERM/ALTYPEAHD/PASSALL  MICOM_PORT  ???

STAT.COM         output results of all possible SHOW commands to MICOM.STAT

MICOM_START.COM  starts MICOM_CONTROL detached process

MICOMINIT.COM    initializes ALL parameters in Micom if they get screwed up.

Standard battery backup of Micom means we have never used this.

T.COM            test SETTERM

Useful hints for debugging:

DUMP/RECORD  *.DAT  to verify completeness

a common cause of 'duplicate key' error during RUN TERMLIST  is blank lines

in TERMLIST.SRC

Should modify TERMINALS and LOCATE to allow CTRL-C to abort.

safe any time except when actually talking to a switch.

TERMLIST.SRC  is the master list of connections

TERMLIST.FOR  reads it and outputs several .DAT files

SETTERM.FOR   when run at Login, talks to both switches and determines

which terminal must have logged in and sets up a

symbol which the system SYSLOGIN.COM uses in a SET TERMINAL .

Users or the manager may type LOCATE to get a table of all current connections

to their current computer, or my use TERMINALS to list all connections

whether completed to a computer or not.  This is slow since

I read the whole table from both switches before starting output.

MICOM.FOR  may be used to talk to the MICOM via the MICOM_CONTROL detached

process.

STAT.COM can be used to record all settings so you can restore all the

'ad hoc' settings you may have made (in the unlikely event that

the Micom loses its mind)  or just to print or review your settings.

FREELINES   is used to kick people off if they clog our homebrew stat mux

but are not logged into any computer.