PCROUTE - an IP routing program from the IBM PC

Vance Morrison

morrison@accuvax.nwu.edu

Traditionally IP routers have been fairly high performance,

expensive machines.  Typically they run about $5000-$10,000 a unit.

Until now a IP router for under $5000 was just about impossible

to get.  Recent developments in PC hardware, however, has made

it possible to convert a PC to an IP router for a TOTAL of $800

a unit.  This price is less that the cost of many ethernet boards

and thus it now makes sense to always use dedicated router to

perform IP gatewaying functions.

---------------------------------------------------------------------

What is PCroute?

PCroute is software written for an PC/XT (or AT) or clone that will

allow it to act as a IP router.   At present the following interfaces

are supported.

Ethernet - (WD8003E card) (recommeded no more than 4 interfaces)

Starlan  - (WD8003S card) (up to 6 interfaces)

Localtalk - (Apple localtalk card for the PC) (1 interface max)

SLIP      - (COM1..COM4)  (2 interfaces max)

One of the most common configuration for the router is as an

ethernet-ethernet router, but this is not the only configuration possible.

The software supports up to 6 interfaces of varying types, so many

many configurations are possible.  Exact details on what is possible

is explained in the installation/compilation manual

In addition to the XT, the only other hardware needed are the

networking cards, which at present run about $225 a piece (for ethernet).

Since you can buy an XT (10Mhz, without an monitor) for $350, the total

cost for the hardware is $800

---------------------------------------------------------------------

What do I need to install PCroute?

1) An XT computer (does not need monitor) with a floppy

2) The proper interface cards

Ethernet : WD8003E              Ethercard Plus

Starlan  : WD8003S,WD8003SH     Starcard,Starlink Plus

Localtalk: Apple localtalk PC card

SLIP     : COM[1-4] ports

---------------------------------------------------------------------

How Fast is PC route?

Some may argue that a PC simply is not fast enough to be a

good IP router.  For slow networks (Localtalk SLIP) this is not really

an issue.  For Ethernet it is a real concern.  Luckily, in the case

of Ethernet (and starlan), the Western Digital cards do a lot of the work.

All the input packet queuing is done by the card, freeing the PC to do the

routing task.   By programming in assembler and optimizing for peak

efficiency (the main loop has NO procedure calls), the PC is up to the task.

Actual tests indicate that that following formula is a worst case

estimate of the throughput of PCroute on a 4.77Mhz XT (based on actual

measurments).

packet_delay = .51 + .00406 * len      msec

Where 'len' is the length of the packet in bytes.  Thus PCroute has a

fixed per packet overhead of .51 msec and takes .00406 msec/byte to transfer

the packet from one network to the other.

Thus for the largest packet size (1514) we get throughput of

packet_delay = .51 + .00406 * 1514 = 6.65 ms

throughput = len*8/packet_delay = 1.8 Mbit/sec

For the smallest packet size (64) we get a throughput of

packet_delay = .51 + .00406 * 64 = .77 ms

throughput = len*8/packet_delay = .66 Mbit/sec

If you are going to by the XT needed, please buy a clone (without

a monitor) with a 10Mhz CPU speed (cost < $350).  This will almost

double the throughput measured above.  If you need more speed you can

go with an AT clone and a even faster CPU speed.  Here are some

actual measurements of packet speeds for some common machines.

per packet       per byte     Packet rate    Throughput

delay           delay        (64bytes)     (1500 bytes)

-----------------------------------------------------------------------------

4.77 Mhz XT    | .51 msec       .00406 msec     1300 /sec    1.8 Mbits/sec

10 Mhz XT      | .237 msec      .00254 msec     2500 /sec    3.0 Mbits/sec

6 Mhz AT       | .169 msec      .00228 msec     3200 /sec    3.3 Mbits/sec

12 Mhz AT (est)| .100 msec      .00220 msec     4150 /sec    3.5 Mbits/sec

16 Mhz AT      | .050 msec      .00190 msec     5800 /sec    4.1 Mbits/sec

As you can see, at the high end, the PCrouter can sustain a thoughput

of close to half the BANDWIDTH of a ethernet (remember 5Mbits of sustained

load is a HEAVY load for an ethernet).  Also remember that this is packets

routed THOUGH the router.  Thus even if 4/10 of the ethernet bandwidth is

being used (that is your ethernet is HEAVILY loaded) and ALL that traffic

is going though the router, PCroute can still keep up.

Note also that the throughput measurements are based on packet delay.

This tends to underestimate the thoughput of the router since it ignores

the fact that there are in fact three processes in the router that are

doing work concerently.  Thus the router can actually handle slightly more

traffic than what the above figure would indicate.

In addition the Ethernet boards have an on-board 6.5K packet input buffer.

Thus packets that come at the PCrouter too fast for it to process will be

queued.  This queueing happens on board and can keep up with the maximum

10Mbit/sec bit rate.  Thus the router will start dropping packets only

after this 6.5K buffer is exhausted.

Note that since SUN NFS likes to send 8K blocks in fast spurts

this will sometimes cause the router to drop packets (since this is

larger than the input queue buffer size).  If you are running NFS through

the router, I suggest that you set the NFS block size down to 4K for

file systems that are mounted though the router (look for 'rsize' and

'wsize' in man fstab).  We use NFS quite heavily here at NU and as

long as rwize and wsize are set to 4K we have no problem.

---------------------------------------------------------------------

What about SLIP speeds?

PCroute also supports up to 2 serial lines in addition to the other

interfaces.  These lines can operate at all the common baud rates

up to 19.2K.  In PCroute with a faster processor (10Mhz XT or an AT clone)

can handle 38.4K or even 57.6K.   All of this using the standard 8250

serial ports.

Note because PC serial ports interupt the processor on EVERY character

SLIP consumes a fair amount of the CPU.  On a 4.77MHZ XT the interupts

for two SLIP ports running at 19.2K Baud consume about 1/2 the available

CPU time.  (actually measurements show that two slip lines running full

tilt at 19.2K consume slightly less than 1/2 the CPU).  Thus packet delays

will double.  Obviously the situation gets better as CPU speed increases.

On a 12Mhz AT interupts consume less than 1/10 of the CPU.

---------------------------------------------------------------------

What is localtalk support?

PCroute supports IP inside of localtalk (appletalk).  This allows

you to send IP packets though localtalk nets.  Since localtalk can

be sent over twisted pair (phonenet) large distances (5000 ft with

a star controler), this method has many of the advantages of SLIP

and is a higher speed solution.  In addition any MACs on the localtalk

network that have MacTCP can access the IP network.

Note, however that PCroute does NOT support encapuslating appletalk

in IP (thus Ushar and CAP will not work), and does NOT support dynamic

IP address assignement.

---------------------------------------------------------------------

What PC route supports?

PCroute was designed to be a fully functional IP router.  In

particular it supports

1) IP routing with Subsets (however the subnet mask

must begin with 255.255)

2) Static routing with up to 250 routes.

3) responds to ICMP echo (ping)

4) Sends ICMP TTL, Redirect, Unreachable when appropriate

5) Fragmentation where necessary

6) RIP dynamic routing protocol

7) Up to 6 interfaces of varying types

8) Error logging using BSD syslog

9) Optional proxy ARP

10) Bootp forwarding

Note that although the software supports up to 6 interfaces, the

total throughput of the router is fixed by the speed of the processor.

For fast boards (ethernet) this gets excessive after 4 interfaces

are installed.

---------------------------------------------------------------------

What PC route DOESN'T support?

1) Ethernet cards besides WD8003E.  It is possible to write

the driver code for other cards, but I personally have no

desire to do so.

---------------------------------------------------------------------

Wish List

These are things that I would like to add to PCroute, but it looks like

I will not have the time.

1) SNMP support.  Everybody wants SNMP support.  Well there is

nothing that says PCroute can't have it.  I have written the

code with this support in mind.  In addition, some people at CMU

have already written the hard part of SNMP and all that would

be necessary is to graft that code onto the various databases

inside PCroute.   I think this would take about 1-2 months of

half time progamming effort.

2) Any other interface that someone might what (T1 or X.25 for

example).   PCroute has been designed to make this addition

relatively simple.

---------------------------------------------------------------------

Hints

1) We found that the 10Mhz XT clones that Jamco and others sell

work very well.  One nice feature about these units is their

BIOS.  By setting the dip switches in the PC, you can tell it

that there is no Monitor.  This also tells the BIOS not to

check for a keyboard either.  Thus you don't need to buy either

the keyboard or the monitor.  Other XT BIOS ALWAYS check for

the keyboard, and thus you have to plug it in even though you

never use it.

---------------------------------------------------------------------

Reliability

The reliability of PCroute has been EXCELLENT.  We have been

using these routers for months now in ten places with absolutely

no failures.   If you wish to PING one for yourself here are some

PC routers on our campus

129.105.49.13

129.105.1.1

129.105.7.1

---------------------------------------------------------------------

Comments and Bug reports

I am interested in finding out what you think of PCroute and

how well it performs for you.  I am also interested in hearing

about any problems you have or bugs in the documentation.

You should send your comments to

Vance Morrison <morrison@accuvax.nwu.edu>

Note that since I am not paid to support this software, I can

not guarantee that I can respond to your problem, but I will

try.

Vance Morrison

Northwestern University

#######################################################################################################################################################################################################################################################################################################################################################################################################################################################################