File:

FILE

ORGANIZATION		sequential

RECORD

FORMAT			undefined

CARRIAGE_CONTROL	fortran

===============================================================================

$ DIR/FULL ROSES_SEQ_UND_FOR.DAT;

Directory DISK3:[DELEYD.ROSES.SEQ]

ROSES_SEQ_UND_FOR.DAT;2                   File ID:  (26459,4,0)

Size:            1/3          Owner:    [SCN,DELEYD]

Created:  23-MAR-1993 18:00:30.63

Revised:  23-MAR-1993 18:00:30.82 (1)

Expires:   <None specified>

Backup:    <No backup recorded>

File organization:  Sequential

File attributes:    Allocation: 3, Extend: 0, Global buffer count: 0, No version limit

Record format:      Undefined, maximum 18 bytes

Record attributes:  Fortran carriage control

RMS attributes:     None

Journaling enabled: None

File protection:    System:RWED, Owner:RWED, Group:RWED, World:RWED

Access Cntrl List:  None

Total of 1 file, 1/3 blocks.

===============================================================================

$ DUMP/HEADER/FORMAT/BLOCKS=COUNT:0 ROSES_SEQ_UND_FOR.DAT

Dump of file DISK3:[DELEYD.ROSES.SEQ]ROSES_SEQ_UND_FOR.DAT;2

File ID (26459,4,0)   End of file block 1 / Allocated 3

File Header

Header area

Identification area offset:           40

Map area offset:                      100

Access control area offset:           255

Reserved area offset:                 255

Extension segment number:             0

Structure level and version:          2, 1

>>> File identification:                  (26459,4,0)

Extension file identification:        (0,0,0)

VAX-11 RMS attributes

>>>     Record type:                      Undefined

>>>     File organization:                Sequential

>>>     Record attributes:                FORTRAN carriage control

Record size:                      18

Highest block:                    3

>>>     End of file block:                1

>>>     End of file byte:                 64

Bucket size:                      0

Fixed control area size:          0

Maximum record size:              0

Default extension size:           0

Global buffer count:              0

Directory version limit:          0

File characteristics:                 <none specified>

Map area words in use:                2

Access mode:                          0

File owner UIC:                       [SCN,DELEYD]

File protection:                      S:RWED, O:RWED, G:RWED, W:RWED

Back link file identification:        (680,19,0)

Journal control flags:                <none specified>

Active recovery units:                None

Highest block written:                3

Identification area

>>> File name:                            ROSES_SEQ_UND_FOR.DAT;2

Revision number:                      1

Creation date:                        23-MAR-1993 18:00:30.63

Revision date:                        23-MAR-1993 18:00:30.82

Expiration date:                      <none specified>

Backup date:                          <none specified>

Map area

Retrieval pointers

Count:          3        LBN:     319002

Checksum:                                 8739

===============================================================================

$ DUMP ROSES_SEQ_UND_FOR.DAT;

Dump of file DISK3:[DELEYD.ROSES.SEQ]ROSES_SEQ_UND_FOR.DAT;2

File ID (26459,4,0)   End of file block 1 / Allocated 3

Virtual block number 1 (00000001), 512 (0200) bytes

202C6465 72206572 61207365 736F5220  Roses are red,  000000

65756C62 20657261 20737465 6C6F6956 Violets are blue 000010

74656577 73207369 20726167 7553202C , Sugar is sweet 000020

21756F79 20657261 206F7320 646E4120  And so are you! 000030

00000000 00000000 00000000 00000000 ................ 000040

File displays as one continuous stream.  Note the spaces at the beginning

of each record:

' 'Roses are red, Violets are blue, Sugar is sweet And so are you!

On an RMS $GET operation, bytes are read in from the file and placed in

the user input buffer pointed to by RAB$L_UBF until the buffer is full

(the size of which is indicated by RAB$W_USZ) or until the the end of

the file is reached.  If the end of file is not reached then the next

RMS $GET operation will continue reading bytes from the file where the

previous one left off.

The file header indicates the end of file is byte 64 of block 1.

===============================================================================

$ ANALYZE/RMS_FILE/INTERACTIVE ROSES_SEQ_UND_FOR.DAT;

FILE HEADER

>>>	File Spec: DISK3:[DELEYD.ROSES.SEQ]ROSES_SEQ_UND_FOR.DAT;2

>>>	File ID: (26459,4,0)

Owner UIC: [SCN,DELEYD]

Protection:  System: RWED, Owner: RWED, Group: RWED, World: RWED

Creation Date:   23-MAR-1993 18:00:30.63

Revision Date:   23-MAR-1993 18:00:30.82, Number: 1

Expiration Date: none specified

Backup Date:     none posted

Contiguity Options:  none

Performance Options: none

Reliability Options: none

Journaling Enabled:  none

ANALYZE> DOWN

RMS FILE ATTRIBUTES

>>>	File Organization: sequential

>>>	Record Format: undefined

>>>	Record Attributes:   fortran

Maximum Record Size: 0

Longest Record: 18

Blocks Allocated: 3, Default Extend Size: 0

>>>	End-of-File VBN: 1, Offset: %X'0040'

File Monitoring: disabled

Global Buffer Count: 0

ANALYZE> DOWN

DATA BYTES (VBN 1, offset %X'0000'):

7  6  5  4  3  2  1  0          01234567

------------------------          --------

61 20 73 65 73 6F 52 20|  0000  | Roses a|

20 2C 64 65 72 20 65 72|  0008  |re red, |

20 73 74 65 6C 6F 69 56|  0010  |Violets |

65 75 6C 62 20 65 72 61|  0018  |are blue|

20 72 61 67 75 53 20 2C|  0020  |, Sugar |

74 65 65 77 73 20 73 69|  0028  |is sweet|

20 6F 73 20 64 6E 41 20|  0030  | And so |

21 75 6F 79 20 65 72 61|  0038  |are you!|

00 00 00 00 00 00 00 00|  0040  |........|

%ANLRMS-I-RESTDONE, All structures at this level have been displayed.