Setup of FD: remote disks

This is a sketch of how to set up the remote disks.

Assuming node HERE:: is where you want the disk to appear to be

and node FAR:: is where the disk actually exists.

0. Edit VMS$V5=1 into all macro images if using VMS V5.x

1. Assemble and link

$macro fddrv

$macro fdhostremot

$Fortran/nocheck/nodebug fdremsub

$Fortran/nocheck/nodebug fdremsrv

$link/nomap fddrv+sys$system:sys.stb/sel+sys$input/option

base=0

$link/nomap/notrace fdhostremot+fdremsub+sys$system:sys.stb/sel

$link/nomap/notrace fdremsrv

Copy fddrv.exe to sys$system: on node HERE::

2. Create sys$system:fdhostremot.cld from fdhost.cld by replacing the

IMAGE name FDHOST by FDHOSTREMOT (in sys$system).

copy FDHOSTREMOT.CLD and .EXE into sys$system: in node HERE::

copy FDREMSRV.EXE onto sys$system: in node FAR::

on node HERE::

3. Install FDHOSTREMOT.EXE with privs CMKRNL, LOG_IO, PHY_IO, NETMBX,

TMPMBX.

On node FAR::

4. Install FDREMSRV.EXE on the remote machine (where the actual disk is)

with privs LOG_IO, NETMBX.

On node HERE::

5. Create a file sys$system:FDH.COM (name not important) that looks

a bit like this (edit as needed):

$assign FAR fdnode

$Assign FDREMSRV fdobj

$Set command sys$system:fdhostremot

$fdhostremot fda0:

6. run sysgen and load FDDRV like this

sysgen>CONNECT FDA0:/NOADAPTER/MAXUNITS=1/DRIVER=FDDRV.EXE

On node FAR::

7. Create a file sys$system:FDREMSRV.COM that reads like this:

$set noon

$assign dua0: FDRDSK

$assign 1 NOWRITE

$! Only assign anything to NOWRITE if you want to ensure that the disk

$! FDRDSK will never be written to. Assign whatever disk you want to

$! have remote mounted as FDRDSK.

$run sys$system:FDREMSRV

$logout/brief

8. Create a DECnet remote object FDREMSRV

like:

ncp>set object FDREMSRV number 0 file sys$system:fdremsrv.com

On node HERE::

9. Start up a command file to run the FD: host. It should look something

like

$run/detach/input=sys$system:FDH.COM/output=nla0: sys$system:loginout.exe

and will run a new process that will contain DCL and will be able to

run FDHOSTREMOT connecting to FDA0: (or whatever unit you've set up).

10. Now fda0: should show up as online to a SHOW DEV FDA0: (wait long

enough for the DECnet circuits to initialize). Once it does, you

can just mount/nowrite fda0: as a regular disk. (mount without the

/nowrite if the remote allows writing.)

If this fails, your DECnet input timer and output timer may not be

long enough. Try setting them to 120 or some such and try again.

The time needed depends on network connections, speeds, and loads

on your system and the remote.

Another IMPORTANT note: make sure your network block count is 17 or

more for DECnet.

$set rms_default/net=20/system

will fix this. FDDRV assumes that you can send/receive DECnet packets

of 8192+24 bytes in one QIO$ and you'll get lots of weird stuff

like invalid buffer length if your system won't allow DECnet i/o

of this length.